NEW SITE OR BIG CHANGE

New Site Traffic Safety Checklist

Use it before a new site opens, and again in the first weeks. Tick what is done, note what is not, and give each gap an owner and a date.

| Site and opening date |  |
| --- | --- |
| Completed by |  |
| Date |  |
| Planned vehicles (type and number) and peak hours |  |

| 1. Layout: people and vehicles apart | Done | To do | Notes, who, by when |
| --- | --- | --- | --- |
| Pedestrian routes separate from vehicle routes, ideally entirely | ☐ | ☐ |  |
| Walkways follow the paths people will naturally take (car park, canteen, office) | ☐ | ☐ |  |
| Crossings at straight stretches with good visibility, not at bends or doors | ☐ | ☐ |  |
| Vehicle doors and gates separated from pedestrian routes that lead onto them | ☐ | ☐ |  |

| 2. Vehicle movements | Done | To do | Notes, who, by when |
| --- | --- | --- | --- |
| One-way route, so less reversing | ☐ | ☐ |  |
| Drive-through loading or parking where possible; reversing areas marked and kept clear | ☐ | ☐ |  |
| Speed limits set, signed and suited to the layout | ☐ | ☐ |  |

| 3. Loading bays and visiting drivers | Done | To do | Notes, who, by when |
| --- | --- | --- | --- |
| A way to stop lorries pulling away during loading (keys, restraint or lights) | ☐ | ☐ |  |
| A safe place for drivers to wait during loading | ☐ | ☐ |  |
| Site rules and a map for visiting drivers at the gate | ☐ | ☐ |  |

| 4. Lighting and conditions | Done | To do | Notes, who, by when |
| --- | --- | --- | --- |
| Routes, yards, crossings and junctions lit; checked after dark | ☐ | ☐ |  |
| Surfaces even, drained and free of obstructions | ☐ | ☐ |  |

| 5. Paperwork for this site | Done | To do | Notes, who, by when |
| --- | --- | --- | --- |
| Risk assessment written for this site's layout and vehicles | ☐ | ☐ |  |
| Traffic management plan drawn on this site's plan | ☐ | ☐ |  |
| Emergency routes and assembly point kept clear of vehicle routes | ☐ | ☐ |  |

| 6. People | Done | To do | Notes, who, by when |
| --- | --- | --- | --- |
| Every driver trained and authorised for the vehicles they will use here | ☐ | ☐ |  |
| Everyone briefed on the new routes, including agency staff and contractors | ☐ | ☐ |  |
| Near-miss reporting ready from the first shift | ☐ | ☐ |  |

Actions

| Gaps to close before opening | Who | By when | Done |
| --- | --- | --- | --- |

Opening review booked (2–4 weeks after opening): ☐   Date: ______________