PORTS

Port Vehicle and Pedestrian Safety Check

A walk round the yard, quay and ramp to check your traffic controls work on this shift. Watch real operations, not a quiet moment. Record what you saw, not just that you went.

| Port / terminal and areas checked |  |
| --- | --- |
| Checked by |  |
| Date, start and end time |  |
| Shift and conditions (day or night, tide, weather, vessel or sailing) |  |

| 1. Pedestrian routes and crossings (yard and quay) | OK | ✗ | What you saw (time, place) |
| --- | --- | --- | --- |
| Walkways avoid cargo handling areas, or are protected where they cross (L148 para 58) | ☐ | ☐ |  |
| Routes marked, signs visible, people using the crossing points (para 71) | ☐ | ☐ |  |
| Nobody on foot in stacks or working areas without a safe system of work (para 70) | ☐ | ☐ |  |
| Barriers in place where vehicles run near an open quay edge (para 62) | ☐ | ☐ |  |

| 2. Reversing and blind spots | OK | ✗ | What you saw (time, place) |
| --- | --- | --- | --- |
| No reversing where a one-way route or drive-through exists (para 85) | ☐ | ☐ |  |
| People on foot clear of reach stackers, straddle carriers and terminal tractors | ☐ | ☐ |  |
| Cameras, mirrors and alarms on large machines clean and working | ☐ | ☐ |  |
| Speed limits followed (para 83) | ☐ | ☐ |  |

| 3. Visiting drivers | OK | ✗ | What you saw (time, place) |
| --- | --- | --- | --- |
| Site rules given to drivers on arrival (para 84) | ☐ | ☐ |  |
| Drivers in the safe area, or the cab if that is the rule, during loading (para 72) | ☐ | ☐ |  |
| Nobody left in a haulage cab on the straddle carrier grid (para 67) | ☐ | ☐ |  |
| Drivers on foot only on marked routes | ☐ | ☐ |  |

| 4. Ro-ro ramp and linkspan | OK | ✗ | What you saw (time, place) |
| --- | --- | --- | --- |
| Protected walkway in use, or people and vehicles kept off the ramp together (para 59) | ☐ | ☐ |  |
| Ship and shore agreed who controls traffic for this vessel (para 60) | ☐ | ☐ |  |
| Vehicles not involved kept out of the loading area (para 63) | ☐ | ☐ |  |
| Ramp surface sound and gripping, slope within its design (para 65) | ☐ | ☐ |  |

| 5. Lighting after dark (do a night check at least every other time) | OK | ✗ | What you saw (time, place) |
| --- | --- | --- | --- |
| Pedestrian and vehicle routes well lit, no lights out (para 234) | ☐ | ☐ |  |
| Obstacles and hazards made conspicuous by lighting or marking (para 235) | ☐ | ☐ |  |

| 6. This shift | OK | ✗ | What you saw (time, place) |
| --- | --- | --- | --- |
| Each control above seen working on this shift, not carried over from the last check | ☐ | ☐ |  |
| Anything new: vessel type, layout, machines, contractors | ☐ | ☐ |  |
| Near misses recorded since the last check (number): ____ | ☐ | ☐ |  |

Actions

| Action for each ✗ | Who | By when | Done |
| --- | --- | --- | --- |

Last check's actions followed up: ☐     Next check (different shift, sailing or tide): ______________

Free check sheet from robok.ai/guides/port-vehicle-pedestrian-safety-check · Based on HSE's Safety in docks (L148), 2014. Paragraph numbers in brackets.